RUBIX CUBE SOLVER

Technical Architecture & Hardware Documentation

ESP32  |  A4988  |  6x NEMA17  |  Python Flask  |  WebSocket

Version 1.0  |  2025

# 1. System Overview

The Rubix Cube Solver is a fully automated electromechanical system that captures the state of an unsolved Rubix cube through a camera, computes the optimal solution algorithmically, and physically executes each move using stepper motors. The system is divided into three decoupled layers.

| LAYER 1 — Frontend | LAYER 2 — Backend | LAYER 3 — Hardware |
| --- | --- | --- |
| Plain HTML / JavaScript Camera capture, Color verification UI, WebSocket client, Progress display | Python + Flask OpenCV color detection, kociemba solver, Move string formatter, REST endpoints | ESP32 + A4988 + NEMA17 WebSocket server, Move parser, Sequential motor controller, Pin mapping |

## Communication Flow — End to End

The data pipeline from camera to cube movement follows this strict sequence:

- User opens website on phone or laptop browser on the same WiFi network as ESP32.

- Camera captures each of the 6 faces one at a time — guided by on-screen prompts.

- Each face image is sent via HTTP POST to the Python Flask backend (/detect endpoint).

- Backend uses OpenCV to identify the 9 sticker colors on the face and returns a JSON array.

- Website displays detected colors in a 3x3 grid — user APPROVES or EDITS each face.

- After all 6 faces are approved, website POSTs the full 54-character facelet string to /solve.

- Backend calls kociemba.solve() and returns the move sequence as a formatted JSON array.

- Website connects to ESP32 WebSocket server and sends the full move array in one packet.

- ESP32 buffers the entire move sequence, then executes each move sequentially.

- ESP32 sends progress updates back via WebSocket — website displays live status.

# 2. Hardware Architecture

## 2.1  Component List

| Component | Specification | Quantity | Purpose |
| --- | --- | --- | --- |
| NEMA17 Stepper Motor | 1.8 deg step / 200 steps/rev | 6 | 1 per face |
| A4988 Stepper Driver | 12-36V VMOT / 3.3-5V logic | 6 | 1 per motor |
| ESP32 Dev Board | Dual-core 240MHz / WiFi+BT | 1 | Main controller |
| 12V 10A SMPS Power Supply | Switching, regulated | 1 | Motor power rail |
| Buck Converter Module | 12V to 5V / 2A min | 1 | ESP32 power |
| 100uF Electrolytic Capacitor | 35V rating or higher | 6 | Spike protection per driver |
| Jumper Wires + PCB | 22 AWG solid core recommended | As needed | Connections |

## 2.2  Power Architecture

WARNING: Never power the ESP32 from the motor power rail directly. Motor switching causes voltage spikes that will damage or reset the ESP32 unpredictably.

Use two completely separate power paths sharing only a single common GND reference point:

| Rail | Voltage / Current | Powers |
| --- | --- | --- |
| Motor Power Rail | 12V DC / 10A | All 6x A4988 VMOT pins |
| Logic Power Rail | 5V DC / 1A (Buck output) | ESP32 VIN pin |
| A4988 Logic (VCC) | 3.3V (from ESP32 3V3 pin) | All 6x A4988 VCC pins |

CAPACITOR RULE: Solder one 100uF capacitor directly across VMOT and GND pins of EACH A4988 driver. This absorbs back-EMF spikes from the stepper coils and is the most common reason A4988s get destroyed.

## 2.3  A4988 Driver — Pin Wiring Per Module

Each of the 6 A4988 modules is wired identically. The following table shows every pin connection:

| A4988 Pin | Connect To | Notes |
| --- | --- | --- |
| VMOT | 12V Motor Rail (+) | Motor coil power — needs capacitor to GND |
| GND (motor side) | 12V Rail GND / Common GND | Common ground point with all other GNDs |
| VCC | ESP32 3V3 pin | Logic supply — 3.3V from ESP32 |
| GND (logic side) | Common GND | Must share ground with ESP32 GND |
| STEP | ESP32 GPIO (unique per motor) | 1 rising pulse = 1 microstep |
| DIR | ESP32 GPIO (unique per motor) | HIGH = Clockwise, LOW = Counter-clockwise |
| ENABLE | ESP32 GPIO G5 (shared ALL) | LOW = motors enabled. HIGH = motors free spin. Wire all 6 to one ESP32 pin. |
| SLEEP | 3.3V (always HIGH) | Pull HIGH to keep driver active always |
| RESET | 3.3V (always HIGH) | Pull HIGH — never leave floating |
| MS1 | 3.3V (HIGH) | Microstepping config — see table below |
| MS2 | 3.3V (HIGH) | Microstepping config — see table below |
| MS3 | GND (LOW) | Microstepping config — 1/8 mode selected |
| 1A, 1B, 2A, 2B | Stepper Motor Coil Wires | Connect motor coil pairs — check datasheet for your specific motor wire order |

## 2.4  Microstepping — Configuration

| MS1 | MS2 | MS3 | Mode | Steps for 90 deg |
| --- | --- | --- | --- | --- |
| LOW | LOW | LOW | Full Step | 50 |
| HIGH | LOW | LOW | 1/2 Step | 100 |
| LOW | HIGH | LOW | 1/4 Step | 200 |
| HIGH | HIGH | LOW | 1/8 Step RECOMMENDED | 400 |
| HIGH | HIGH | HIGH | 1/16 Step | 800 |

RECOMMENDED: Use 1/8 microstepping (MS1=HIGH, MS2=HIGH, MS3=LOW). 400 steps for 90 degrees gives smooth motion without being so slow that it impacts performance. Wire MS pins the same on all 6 A4988 drivers.

## 2.5  ESP32 GPIO Pin Mapping

Each motor requires 2 dedicated GPIO pins (STEP and DIR). One shared ENABLE pin controls all motors simultaneously.

| Motor / Face | Move Code | STEP Pin | DIR Pin | Rotation (CW=90deg) |
| --- | --- | --- | --- | --- |
| Motor 1 — RIGHT face | R | GPIO 13 | GPIO 12 | DIR = HIGH |
| Motor 2 — LEFT face | L | GPIO 14 | GPIO 27 | DIR = HIGH |
| Motor 3 — UP face | U | GPIO 26 | GPIO 25 | DIR = HIGH |
| Motor 4 — DOWN face | D | GPIO 33 | GPIO 32 | DIR = HIGH |
| Motor 5 — FRONT face | F | GPIO 19 | GPIO 18 | DIR = HIGH |
| Motor 6 — BACK face | B | GPIO 17 | GPIO 16 | DIR = HIGH |
| ENABLE (shared - ALL motors) | — | GPIO 5 | — | LOW = Active |

NOTE: Counter-clockwise rotation (R', L', U', D', F', B') is the same number of steps with DIR pin set LOW instead of HIGH. 180-degree moves (R2) are simply 800 steps (double 90-degree steps).

# 3. Color Detection, Approval & Edit System

Color detection is the most critical software step. An incorrect color reading produces an unsolvable facelet string, causing kociemba to error or the robot to scramble the cube further. This section defines the complete approval and correction workflow.

## 3.1  Detection Pipeline

- User clicks Capture on the website for each face.

- JavaScript captures a still frame from the camera stream using HTML canvas.

- Image is encoded as base64 and POSTed to Flask /detect endpoint.

- Python OpenCV converts image to HSV color space (more robust than RGB for color matching).

- The 9 sticker regions are sampled from a fixed grid template applied to the captured image.

- Each sample is matched against predefined HSV ranges for all 6 Rubik colors.

- Backend returns a JSON array of 9 color labels: e.g. ["R","W","G","R","Y","B","O","R","W"].

- Website renders a 3x3 interactive grid with colored squares showing detected result.

## 3.2  Color Approval UI — Keyboard Controls

After each face is detected, the website shows the result in an interactive 3x3 grid. The user must explicitly approve or correct before proceeding to the next face.

| Key / Action | Function | Details |
| --- | --- | --- |
| ENTER / Spacebar | APPROVE current face | Locks the 9 detected colors and advances to next face. Cannot undo after advancing. |
| E (Edit Mode) | Toggle Edit Mode | Enters cell selection mode. Arrow keys move the blue selection highlight across the 3x3 grid. |
| Arrow Keys | Navigate grid cells | Moves selection cursor Up / Down / Left / Right within the 3x3 grid while in Edit Mode. |
| 1 — 6 (Number Keys) | Set sticker color | 1=White 2=Yellow 3=Red 4=Orange 5=Blue 6=Green. Sets the selected cell to chosen color. |
| R (while in Edit Mode) | Re-detect this face | Triggers a new camera capture and re-runs the detection pipeline for the current face only. |
| Escape | Exit Edit Mode | Returns to View Mode without saving pending edits. Press Enter to confirm edits before Escape. |
| Backspace / B | Go back one face | Allowed only before all 6 faces are submitted. Returns to the previous face for re-capture. |

## 3.3  Face Capture Sequence

Faces must be captured in a fixed order so the facelet string is assembled correctly for kociemba. The website enforces this order with on-screen prompts showing which face to hold toward the camera next.

| Step | Face Code | Face Name | Instruction to User |
| --- | --- | --- | --- |
| 1 | U | UP (Top) | Hold cube with White center facing camera |
| 2 | R | RIGHT | Rotate cube — Right face toward camera |
| 3 | F | FRONT | Rotate cube — Front face toward camera |
| 4 | D | DOWN (Bottom) | Rotate cube — Bottom face toward camera |
| 5 | L | LEFT | Rotate cube — Left face toward camera |
| 6 | B | BACK | Rotate cube — Back face toward camera |

# 4. Communication — Website to ESP32

## 4.1  Protocol Selection — WebSocket

WebSocket is chosen over HTTP POST for the final command delivery to ESP32 because it provides a persistent bidirectional connection — meaning the ESP32 can continuously send progress updates back to the browser without the browser needing to poll.

| Method | Speed | 2-Way | Complexity | Verdict |
| --- | --- | --- | --- | --- |
| WebSocket (CHOSEN) | Fast | YES | Medium | BEST |
| HTTP POST | Slow | No | Low | Acceptable |
| MQTT | Medium | YES | High | Overkill |

## 4.2  Network Topology

- Both the laptop/phone running the website AND the ESP32 must be on the same WiFi network (LAN).

- ESP32 starts a WebSocket server on port 81 after connecting to WiFi at boot.

- The Python Flask backend runs on the user's laptop/PC on port 5000.

- The website (HTML/JS) runs locally or served by Flask — it communicates with both Flask and ESP32.

- No internet connection is required once setup is complete. Fully local network operation.

## 4.3  Message Protocol — Defined Packet Types

| Sender | Message | Meaning |
| --- | --- | --- |
| Browser → ESP32 | {"cmd":"SOLVE", "moves":[...]} | Full move array sent once. ESP32 buffers the entire sequence before starting any motor. |
| ESP32 → Browser | {"status":"READY"} | ESP32 sends this after receiving full move array — confirms buffering complete, about to start. |
| ESP32 → Browser | {"status":"MOVE", "n":5, "total":20, "move":"R'"} | Sent after each move executes. Browser updates the progress bar in real time. |
| ESP32 → Browser | {"status":"COMPLETE"} | All moves done. Browser shows success animation. Motors can be de-energized now. |
| ESP32 → Browser | {"status":"ERROR", "msg":"..."} | Unexpected issue. Browser shows error message. Solve is aborted and motors are disabled. |
| Browser → ESP32 | {"cmd":"STOP"} | Emergency stop. ESP32 immediately halts all motors mid-sequence if this is received. |

## 4.4  Move String Format

The kociemba library returns a standard space-separated move string. The Flask backend parses and expands this into a flat JSON array before sending to ESP32:

kociemba output :   "R U2 R' F B' D2"

Flask expansion :   ["R", "U", "U", "R_CCW", "F", "B_CCW", "D", "D"]

EXPANSION RULE: R2 becomes two R entries. R' (prime) becomes R_CCW. This simplifies ESP32 parsing — it only needs to handle two direction variants and never needs to count.

# 5. Sequential Motor Execution

This is the most safety-critical aspect of the firmware. Only one motor may move at any time. Two motors firing simultaneously will physically jam or damage the cube mechanism. The ESP32 firmware enforces strict sequential execution.

## 5.1  Execution State Machine

| State | Description |
| --- | --- |
| IDLE | Waiting for WebSocket command. All motors disabled (ENABLE pin HIGH). Low power consumption. |
| BUFFERING | Receiving move array over WebSocket. Stores all moves in RAM array. Does NOT move any motor. |
| READY | Full sequence buffered. Sends READY message to browser. Enables all motors (ENABLE = LOW). Waits 200ms for coils to energize before first move. |
| EXECUTING | Processing moves one by one. For each move: set DIR, pulse STEP 400 times, wait settle delay, send MOVE progress update, advance index. |
| COMPLETE | All moves processed. Sends COMPLETE to browser. Waits 2 seconds in holding mode then optionally disables motors to reduce heat. |
| STOPPED | STOP command received mid-execution. Halts immediately after current step pulse. Disables motors. Returns to IDLE. |

## 5.2  Step Pulse Parameters

| Parameter | Value | Purpose |
| --- | --- | --- |
| Steps per 90-degree rotation | 400 steps (1/8 mode) | NEMA17 200 steps x 2 microsteps = 400 |
| Steps per 180-degree rotation (X2) | 800 steps | Simply double the 90-degree count |
| Step pulse HIGH duration | 5 microseconds | A4988 minimum STEP pulse width = 1us. 5us is safe. |
| Step pulse LOW duration (delay) | 500 microseconds (adjustable) | Controls rotation speed. Lower = faster. Tune to avoid skipping. |
| Between-move settle delay | 50 - 100 milliseconds | Allows motor to stop vibrating before next move begins. |
| Acceleration ramp | First 40 steps ramp up speed | Start slow, reach full speed mid-rotation. Prevents step skipping at high torque start. |

## 5.3  Move Execution Logic — Pseudocode

FUNCTION executeMoves(moveArray):

digitalWrite(ENABLE_PIN, LOW)          // energize all motor coils

delay(200)                             // wait for coils to stabilize

FOR index = 0 TO length(moveArray) - 1:

move = moveArray[index]

motor = getMotor(move)               // look up STEP/DIR pins

direction = getDirection(move)       // CW or CCW

steps = getSteps(move)               // 400 or 800

IF stopSignal == TRUE: BREAK         // emergency stop check

digitalWrite(motor.DIR, direction)

FOR step = 0 TO steps - 1:

digitalWrite(motor.STEP, HIGH)

delayMicroseconds(5)

digitalWrite(motor.STEP, LOW)

delayMicroseconds(stepDelay)       // 500us default

delay(75)                            // settle between moves

sendWebSocket(MOVE_PROGRESS, index+1, total)

sendWebSocket(COMPLETE)

delay(2000)

digitalWrite(ENABLE_PIN, HIGH)         // optional: de-energize

# 6. Risk Register & Mitigations

| Risk | Severity | Mitigation |
| --- | --- | --- |
| Voltage spike kills A4988 driver | CRITICAL | 100uF capacitor across VMOT/GND on every A4988. Non-negotiable. |
| Two motors fire simultaneously | CRITICAL | Strict sequential firmware loop. No threads or async motor calls. One completes before next starts. |
| Wrong color detection (red vs orange) | HIGH | HSV tuning + mandatory user approval step + full manual edit mode before solving. |
| Step count drift — motor loses position | HIGH | Calibration mode: run 1 motor for 400 steps and verify exactly 90 degrees manually. Add acceleration ramp. |
| WiFi drops mid-solve | MEDIUM | ESP32 buffers entire move array before starting. Solve completes even if WiFi drops after transfer. |
| kociemba receives invalid 54-char string | MEDIUM | Flask validates string before calling solve: check 9 of each color present, center colors correct. |
| ESP32 brownout from motor current | MEDIUM | Separate power rails. Buck converter for ESP32. Never power ESP32 from motor rail. |

# 7. Recommended Build Sequence

| Phase | Focus | Milestone / Acceptance Criteria |
| --- | --- | --- |
| 1 | Single motor + A4988 + ESP32 | One motor rotates exactly 90 degrees in each direction. Verify physically with a protractor. |
| 2 | All 6 motors wired and tested | Each face motor individually confirmed. No pins conflicting. ENABLE shared pin works. |
| 3 | ESP32 WebSocket server | Browser connects to ESP32 IP. Send a test array. Confirm received in ESP32 serial monitor. |
| 4 | Sequential move execution firmware | Send [R, U, R_CCW] — watch cube physically perform three moves in sequence with no overlap. |
| 5 | Flask backend + color detection | Capture one face image. GET correct 9-color JSON response. Test all 6 colors detected reliably. |
| 6 | Frontend — camera + approval UI | Capture all 6 faces. Approval and edit workflow functional. 54-char string assembled correctly. |
| 7 | End-to-end integration test | Manually scramble cube. Full pipeline scan-to-solve works. Cube ends in solved state. |
| 8 | Speed tuning | Reduce step delay incrementally. Find fastest speed without skipping steps under cube load. |

Document prepared by Phoenix  |  Rubix Cube Solver Project

All specifications subject to calibration during physical build phase.